Ardublock by SumoBoy installation

- Check if Arduino IDE is already installed on tour PC

(Default location is: C:\Program Files(x86) \Arduino)

- Download Setup file:

  - If you don’t have Arduino IDE:

https://github.com/AleksandrsKorsunovs/Ardublock-for-Sumoboy/raw/master/ArduBlockSumoBoy%2BArduinoSetup.msi

  - If you have Arduino IDE:

https://github.com/AleksandrsKorsunovs/Ardublock-for-Sumoboy/raw/master/ArdublockSumoboySetup.msi

- Launch Setup file and install

- Ardublock for SumoBoy tool is ready to use

Using Ardublock

- Launch ArduinoIDE

- Connect SumoBoy robot via USB to PC

- Make sure correct PORT and BOARD are selected (Port number is individual for each PC, but it should have “(Arduino/Genuino Micro)”)

- Create and save new project

- Launch Ardublock tool

- Create some BLINK program

  - Select “SumoBoy” drawer (Left click)

  - Find and drag to workspace “Turn Led” block

  - Connect it to “Program” block’s LOOP connector

  - Set LED and its start STATE

  - Select “Control” drawer

  - Find and drag to workspace “delay MILLIS” block

  - Connect it to “Turn Led” block

  - Type in “delay MILLIS” interval in millisecond (1000 ms = 1 s) (Left click on “Number” block -> Type -> press “ENTER”)

  - Type that little formula in comment (Right click on block-> Comment -> Type comment)

  - Clone “Turn Led” block and “delay MILLIS” block (Right click on “Turn Led” block-> Clone. Clones selected block and all block connected to it)

  - Connect copied blocks to “delay MILLIS” block

  - Change STATE in copied “Turn Led” block to opposite

  - If you are not sure what block or connector does, hover mouse over it – most blocks have description, which appears on hover

- Program is ready. To upload it to SumoBoy press “Upload To Arduino” button (Make sure PORT and BOARD are selected correctly)